PW2: Spectral responses

starting from true or false colour images

Objectives :

- How to record radiometric values for different types of landscapes (ground, water, vegetation, urban areas,…) in the spectral bands of blue, green, red and NI (near infrared).

- How to encode these values in a spreadsheet

- How to make a graphic with the radiometric values according to the spectral bands.

Manipulations :

1°) Load the software “Leoworks”

2°) Open the images of Cairo in bands 1, 2,  3 and 4 of Landsat 1982. All the images can be found in the file “Data” within the file “Leoworks”. Do not use the function “Enhance”, because it modifies the radiometric values.

3°) Make a false colour image in order to see where you are (your location) and then a real colour image.

4°) Record the radiometric values for different types of landscapes in the image in true and false colours or else in each image (band 1, band 2, …)

Choose “Cursor Position” in the menu “View” in order to visualise the radiometric values of the spot where the cursor is positioned.

Radiometric value for the pixel of which the coordinates are X=231 and Y=5. If the image is georeferenciated the geographical coordinates are also available / these are also the geographic coordinates

5°) Open a spreadsheet  in order to encode the different radiometric values.

| types | V (Green) | R (Red) | IR (NIR) |
| --- | --- | --- | --- |
| water | 61 | 44 | 50 |
| forest | 53 | 38 | 92 |
| areas under cultivation | 59 | 34 | 196 |
| ploughed fields | 81 | 74 | 129 |
| road | 69 | 59 | 123 |
| white soil (gypsum) | 172 | 155 | 181 |

For example:

Remark :

In a combined image (Combined Image), it’s also possible to view the three radiometric answer (directly):

Position of the pixel (192,283)

Radiometric value for Red (63) ,  Green (68) and Blue (96)