PW 3: Measuring distances on an image

Objectives :

First contact with “Leoworks”.

Using measurement tools for distances, perimeters and surfaces areas.

Manipulations :

1°) Load “Leoworks”

2°) Open the images of Cairo on band 1, 2, 3 and 4 of Landsat 1982. All the images can be found in file “Data” within file “Leoworks”

3°) Use the function “Enhance” for each Landsat image.

4°) Make a true colour combined image and a false colour combined image.

5°) Check the dimension of one pixel in “Pixel Size” in (the) menu “Image”. In this example, the image is georeferenciated and the information given about the image automatically gives 30m by 30m. Click on OK.

6°) Using measurement tools.

a) Choose “Measurement Tool” in (the) menu “Image”

Enlarge the image with the function “magnifying glass +”  on the top of the window of your combined image (in true or false colour).

b) Choose (the) menu “Units” in the window “Measurement tool” and select unit “Meter”  (or pixel)

How to calculate a distance

Click (left with PC) for the starting point of your measurement . Click (left) in order to determine another point. Click (right) to stop the measurement. With Mac equipped with PC emulator “ctrl” + “click” = “click” right. A measure of a line segment is displayed (1230 meters for the Nile river, right before the island). Work in the same way to measure a surface area or a perimeter, but do not stop the measurement after the 2d point: just click on a 3d point, then on a 4th, etc.; click right in order to complete the measurement.

# Measure the width of the Nile river at different places

Measure the perimeter and the surface area of the island that lies in the center of the image of Cairo.

Further applications:

- Measure the diameter and the surface area of moon or mars craters.

- Measure the length of a dam and compare your result with its official length; calculate the % of uncertainty of your measure.